DEPARTMENT OF CSE

LAB MANUAL

ACADEMIC YEAR: 2022-23

(2019 Batch - Regulation 2020)

Programme(UG/PG) : UG

Semester : III B.Tech., I Semester

Course Code: CS3105

Course Title : COMPUTER NETWORKS LAB

Prepared By

Dr. Syed Sadat Ali,

Professor,

Dept. of CSE, NCET.

NIMRA COLLEGE OF ENGINEERING & TECHNOLOGY(23)

NIMRA NAGAR, JUPUDI, IBRAHIMPATNAM-521 456.

(Approved by AICTE, Recognized by Govt. of A.P. Affliated to JNTUK, Kakinada.)

INTERNAL ASSESSMENT MARK SPLIT UP

Observation (Day to Day work) : 10 Marks

Record: 5 Marks

- GENERAL INSTRUCTIONS

- 1.1 General discipline in the lab

  - Students must turn up in time and contact concerned faculty for the experiment they are supposed to perform.

  - Students will not be allowed to enter late in the lab.

  - Students will not leave the class till the period is over.

  - Students should come prepared for their experiment.

  - Experimental results should be entered in the lab report format and certified/signed by concerned faculty/ lab Instructor.

  - Students must get the connection of the hardware setup verified before switching on the power supply.

  - Students should maintain silence while performing the experiments. If any necessity arises for discussion amongst them, they should discuss with a very low pitch without disturbing the adjacent groups.

  - Violating the above code of conduct may attract disciplinary action.

  - Damaging lab equipment or removing any component from the lab may invite penalties and strict disciplinary action.

- 1.2 Attendance

- Attendance in the lab class is compulsory.

- Students should not attend a different lab group/section other than the one assigned at the beginning of the session.

- On account of illness or some family problems, if a student misses his/her lab classes, he/she may be assigned a different group to make up the losses in consultation with the concerned faculty / lab instructor. Or he/she may work in the lab during spare/extra hours to complete the experiment. No attendance will be granted for such case.

- 1.3 Preparation and Performance

- Students should come to the lab thoroughly prepared on the experiments they are assigned to perform on that day. Brief introduction to each experiment with information about self study reference is provided on LMS.

- Students must bring the lab report during each practical class with written records of the last experiments performed complete in all respect.

- Each student is required to write a complete report of the experiment he has performed and bring to lab class for evaluation in the next working lab. Sufficient space in work book is provided for independent writing of theory, observation, calculation and conclusion.

- Students should follow the Zero tolerance policy for copying / plagiarism. Zero marks will be awarded if found copied. If caught further, it will lead to disciplinary action.

- Refer Annexure 1 for Lab Report Format

- LIST OF EXPERIMENTS

| S. No. | Experiment | Page No. |
| --- | --- | --- |
| 1. | Study of Network devices in detail and connect the computers in Local Area Networks. | 6 |
| 2. | Write a Program to implement the data link layer framing methods such as ( i ) Character Stuffing ( ii ) Bit Stuffing. | 11, 14 |
| 3. | Write a Program to implement the data link layer framing methods Checksum. | 16 |
| 4. | Write a Program for Hamming Code Generation for error deduction and correction. | 20 |
| 5. | Write a Program to implement on a data set of characters the three CRC polynomials - CRC 12, CRC 16 and CRC CCIP. | 22 |
| 6 | Write a Program to implement Slidinng window protocol for Goback N . | 25 |
| 7. | Write a Program to implement Slidinng window protocol for Selective repeat. | 28 |
| 8. | Write a Program to implement Stop and Wait Protocol. |  |
| 9. | Write a program for congestion control using leaky bucket algorithm. |  |
| 10. | Write a program to implement Dijkstra’s algorithm to compute Shortes path through a graph. | 32 |
| 11. | Write a Program to implement Distance vector routing algorithm buy obtaining routing table at each node. (Take an example subnet grapoh with weights indicating delay between nodes). | 35 |
| 12. | Write a Program to implement Broadcast tree by taking subnet of nosts. | 38 |
| 13. | Wireshark - Packet Captuire Using wire shark. - Starting Wire Shark. - Viewing Captured Traffic. - Analysis and Statistics & Filters. | 41 47 51 52 |
| 14. | How to run Nimap scan. | 55 |
| 15. | Operating System Detection using Nmap. | 67 |
| 16. | Do the following using NS2 Simulator. i . NS2 Simulator-Introduction. ii . Simulate to Find the Number of Packets Dropped. iii . Simulate to Find the Numnber of Packets Dropped by TCP/UDP. iv . Simulate to Find the Numnber of Packets Dropped to Congestion. v . Simulate to Compare Data Rate & Throughput. |  |

| Marks Distribution |  |
| --- | --- |
| Continuous Evaluation (15 Marks) | End Semester Exam (35 Marks) |
| Each experiment shall be evaluated for 10 marks and at the end of the semester proportional marks shall be awarded out of 10. | End semester practical exam conduction and evaluation will be conducted by the External Examiner appointed by JNTUK, Kakinada.. |
| Following is the breakup of 10 marks for each 4 Marks: Observation & conduct of experiment. Teacher may ask questions about experiment. 3 Marks: For report writing 3 Marks: For the 15 minutes quiz to be conducted in every lab. |  |

EXPERIMENT NO: 1

(1 a)

NAME OF THE EXPERIMENT: Study of Network devices in details and connect the

Computers in Local Area Network.

OBJECTIVE: Study of Network devices used in LAN

RESOURCE : Computer Networks Text Book by Stallings.

Here is the common network device list:

- Hub.

- Switch.

- Router.

- Bridge.

- Gateway.

- Modem.

- Repeater.

- Access Point.

- Switch:A network switch or switching hub is a computer networking device that connects network segments.The term commonly refers to a network bridge that processes and routes data at the data link layer (layer 2) of the OSI model. Switches that additionally process data at the network layer (layer 3 and above) are often referred to as Layer 3 switches or multilayer switches.

- Router: A router is an electronic device that interconnects two or more computer networks, and selectively interchanges packets of data between them. Each data packet contains address information that a router can use to determine if the source and destination are on the same network, or if the data packet must be transferred from one network to another. Where multiple routers are used in a large collection of interconnected networks, the routers exchange information about target system addresses, so that each router can build up a table showing the preferred paths between any two systems on the interconnected networks.

- Hub: An Ethernet hub, active hub, network hub, repeater hub, hub or concentrator

is a device for connecting multiple twisted pair or fiber optic Ethernet devices together and making them act as a single network segment. Hubs work at the physical layer (layer 1) of the OSI model. The device is a form of multiport repeater. Repeater hubs also participate in collision detection, forwarding a jam signal to all ports if it detects a collision.

- Bridge: A network bridge connects multiple network segments at the data link layer (Layer 2) of the OSI model. In Ethernet networks, the term bridge formally means a device that behaves according to the IEEE 802.1D standard. A bridge and switch are very much alike; a switch being a bridge with numerous ports. Switch or Layer 2 switch is often used interchangeably with bridge.Bridges can analyze incoming data packets to determine if the bridge is able to send the given packet to another segment of the network.

- Gate Way: In a communications network, a network node equipped for interfacing with

another network that uses different protocols.

  - A gateway may contain devices such as protocol translators, impedance matching devices, rate converters, fault isolators, or signal translators as necessary to provide system interoperability. It also requires the establishment of mutually acceptable administrative procedures between both networks.

  - A protocol translation/mapping gateway interconnects networks with different network protocol technologies by performing the required protocol conversions.

- Repeater:Functioning at Physical Layer.A repeater is an electronic device that receives

- a signal and retransmits it at a higher level and/or higher power, or onto the other side of

- an obstruction, so that the signal can cover longer distances. Repeater have two ports, so

- cannot be use to connect for more than two devices

- Modem: Modems (modulators-demodulators) are used to transmit digital signals over analog telephone lines. Thus, digital signals are converted by the modem into analog signals of different frequencies and transmitted to a modem at the receiving location. The receiving modem performs the reverse transformation and provides a digital output to a device connected to a modem, usually a computer. The digital data is usually transferred to or from the modem over a serial line through an industry standard interface, RS-232. Many telephone companies offer DSL services, and many cable operators  use modems as end terminals for identification and recognition of home and personal users. Modems work on both the Physical and Data Link layers.

- Repeater: A repeater is an electronic device that amplifies the signal it receives. You can think of repeater as a device which receives a signal and retransmits it at a higher level or higher power so that the signal can cover longer distances, more than 100 meters for standard LAN cables. Repeaters work on the Physical layer.

- Access Point: While an access point (AP) can technically involve either a wired or wireless connection, it commonly means a wireless device. An AP works at the second OSI layer, the Data Link layer, and it can operate either as a bridge connecting a standard wired network to wireless devices or as a router passing data transmissions from one access point to another.

Wireless access points (WAPs) consist of a transmitter and receiver (transceiver) device used 	to

create a wireless LAN (WLAN). Access points typically are separate network devices with a built-

in antenna, transmitter and adapter. APs use the wireless infrastructure network mode to provide a

connection point between WLANs and a wired Ethernet LAN. They also have several ports, giving

you a way to expand the network to support additional clients. Depending on the size of the network,

one or more APs might be required to provide full coverage. Additional APs are used to allow

access to more wireless clients and to expand the range of the wireless network.

Each AP is limited by its transmission range — the distance a client can be from an AP and still

obtain a usable signal and data process speed. The actual distance depends on the wireless standard,

the obstructions and environmental conditions between the client and the AP. Higher end APs have

high-powered antennas, enabling them to extend how far the wireless signal can travel.

APs might also provide many ports that can be used to increase the network’s size, firewall

capabilities and Dynamic Host Configuration Protocol (DHCP) service. Therefore, we get APs that

are a switch, DHCP server, router and firewall.

To connect to a wireless AP, you need a service set identifier (SSID) name. 802.11 wireless

networks use the SSID to identify all systems belonging to the same network, and client stations

must be configured with the SSID to be authenticated to the AP. The AP might broadcast the SSID,

allowing all wireless clients in the area to see the AP’s SSID. However, for security reasons, APs

can be configured not to broadcast the SSID, which means that an administrator needs to give client

systems the SSID instead of allowing it to be discovered automatically. Wireless devices ship with

default SSIDs, security settings, channels, passwords and usernames. For security reasons, it is

strongly recommended that you change these default settings as soon as possible because many

internet sites list the default settings used by manufacturers.

Access points can be fat or thin. Fat APs, sometimes still referred to as autonomous APs, need to be

manually configured with network and security settings; then they are essentially left alone to serve

clients until they can no longer function. Thin APs allow remote configuration using a controller.

Since thin clients do not need to be manually configured, they can be easily reconfigured and

monitored. Access points can also be controller-based or stand-alone.

Conclusion: Having a solid understanding of the types of network devices available can help you design and built a network that is secure and serves your organization well. However, to ensure the ongoing security and availability of your network, you should carefully monitor your network devices and activity around them, so you can quickly spot hardware issues, configuration issues and attacks.

NAME OF THE EXPERIMENT: Connect the Computers in Local Area Network.

OBJECTIVE: Study of Network devices used in LAN

RESOURCE : Computer Networks Text Book by Stallings.

Aim: Connect the computers in Local Area Network.

Procedure: On the host computer

On the host computer, follow these steps to share the Internet connection:

- Log on to the host computer as Administrator or as Owner.

- Click Start, and then click Control Panel.

- Click Network and Internet Connections.

- Click Network Connections.

- Right-click the connection that you use to connect to the Internet. For example, if you

- connect to the Internet by using a modem, right-click the connection that you want

- Under Dial-up / other network available.

- Click Properties.

- Click the Advanced tab.

- Under Internet Connection Sharing, select the Allow other network users to

- Connect through this computer's Internet connection check box.

- If you are sharing a dial-up Internet connection, select the Establish a dial-up

- connection whenever a computer on my network attempts to access the Internet

- check box if you want to permit your computer to automatically connect to the

- Internet.

- Click OK. You receive the following message:

- When Internet Connection Sharing is enabled, your LAN adapter will be set to use IP address

- 192.168.0.1. Your computer may lose connectivity with other computers on your network. If

- these other computers have static IP addresses, it is a good idea to set them to obtain their IP

- addresses automatically. Are you sure you want to enable Internet Connection Sharing?

- Click Yes.

The connection to the Internet is shared to other computers on the local area network (LAN).

The network adapter that is connected to the LAN is configured with a static IP

address of 192.168.0.1 and a subnet mask of 255.255.255.0

On the client computer

To connect to the Internet by using the shared connection, you must confirm the LAN adapter IP configuration, and then configure the client computer. To confirm the LAN adapter IP configuration, follow these steps:

- Log on to the client computer as Administrator or as Owner.

- Click Start, and then click Control Panel.

- Click Network and Internet Connections.

- Click Network Connections.

- Right-click Local Area Connection and then click Properties.

- Click the General tab, click Internet Protocol (TCP/IP) in the connection uses

- the following items list, and then click Properties.

- In the Internet Protocol (TCP/IP) Properties dialog box, click Obtain an IP

- address automatically (if it is not already selected), and then click OK.

Note: You can also assign a unique static IP address in the range of 192.168.0.2

to 192.168.0.254. For example, you can assign the following static IP

address, subnet mask, and default gateway:

8. IP Address 192.168.31.202

9. Subnet mask 255.255.255.0

- Default gateway 192.168.31.1

- In the Local Area Connection Properties dialog box, click OK.

- Quit Control Panel.

EXPERIMENT NO: 2

(2a)

NAME OF THE EXPERIMENT: Bit Stuffing.

OBJECTIVE: Implement the data link layer framing method.

RESOURCE: Turbo C

PROGRAM LOGIC: The new technique allows data frames to contain an arbitrary  number  if  bits and allows character codes with an arbitrary no of bits per character. Each frame begins and ends with special bit pattern, 01111110, called a flag byte. Whenever the sender’s data link layer encounters five consecutive ones in the data ,   i t  a u tomatical ly stuffs a 0 bit into the out going bit stream.  This bit stuffing is analogous to c haracter stuf fin g, in w hich a D LE i s stuf fed into the out going                     character stream before DLE in the data.

SOURCE CODE:

// BIT Stuffing program #include<stdio.h> #include<conio.h> #include<string.h>

void main()

{

int a[20],b[30],i,j,k,count,n; clrscr();

printf("Enter frame length:"); scanf("%d",&n);

printf("Enter input frame (0's & 1's only):"); for(i=0;i<n;i++)

scanf("%d",&a[i]); i=0;

count=1; j=0;

while(i<n)

{

if(a[i]==1)

{

b[j]=a[i];

for(k=i+1;a[k]==1 && k<n &&count<5;k++)

{

j++;

b[j]=a[k]; count++; if(count==5)

{

j++; b[j]=0;

}

i=k;

}

}

else

{

b[j]=a[i];

} i++; j++;

}

printf("After stuffing the frame is:"); for(i=0;i<j;i++)

printf("%d",b[i]); getch();

} OUTPUT:

Viva questions:

- What is Stuffing?

- What is use of Stuffing?

- With bit stuffing the boundary between two frames can be unambiquously recognize by?

- is a analogous to character stuffing?

- The senders data link layer encounters	no of 1’s consecutively

EXPERIMENT NO: 2

2(b)

NAME OF THE EXPERIMENT: Character Stuffing. OBJECTIVE: Implement the data link layer framing methods. RESOURCE: Turbo C

PROGRAM LOGIC:

The framing method gets around the problem of resynchronization after an error by ha vi ng ea ch fra me sta r t with th e ASCII c ha ra cter  s equ en ce  D L E ST X a nd  the sequence DLE ETX. If the destination ever losses the track of the frame boundaries all it has to do is look for DLE STX or DLE ETX characters to figure out. The data link la yer  on  t he recei vin g en d re mo ve s th e D LE b efore th e da ta a re giv en  t o th e network layer. This technique is called character stuffing.

PROCEDURE: Go to debug -> run or press CTRL + F9 to run the program.

SOURCE CODE:

//PROGRAM FOR CHARACTER STUFFING

#include<stdio.h> #include<conio.h> #include<string.h> #include<process.h> void main()

{

int i=0,j=0,n,pos;char a[20],b[50],ch; clrscr();

printf("enter string\n"); scanf("%s",&a); n=strlen(a);

printf("enter position\n"); scanf("%d",&pos); if(pos>n)

{

printf("invalid position, Enter again :"); scanf("%d",&pos);

}

printf("enter the character\n"); ch=getche();

b[0]='d';

b[1]='l';

b[2]='e';

b[3]='s';

b[4]='t';

b[5]='x'; j=6;

while(i<n)

{

if(i==pos-1)

{

b[j]='d';

b[j+1]='l';

b[j+2]='e';

b[j+3]=ch; b[j+4]='d';

b[j+5]='l';

b[j+6]='e';

j=j+7;

}

if(a[i]=='d' && a[i+1]=='l' && a[i+2]=='e')

{

b[j]='d';

b[j+1]='l';

b[j+2]='e';

j=j+3;

}

b[j]=a[i]; i++;

j++;

}

b[j]='d';

b[j+1]='l';

b[j+2]='e';

b[j+3]='e';

b[j+4]='t';

b[j+5]='x';

b[j+6]='\0';

printf("\nframe after stuffing:\n"); printf("%s",b);

getch();

}

OUTPUT:

Viva Questions:

  - What is Character stuffing?

  - What is the use of character stuffing?

  - _ _ _ _ _ _are the delimiters for the character stuffing?

  - Expand DLE STX?

  - Expand DLE ETX?

EXPERIMENT NO: 3

NAME OF THE EXPERIMENT: Program to implement data link layer framing method checksum.

OBJECTIVE:The Checksum is an error detection method that detected errors in data/message while it is transmitted from sender to receiver. This method is used by the higher layer protocols and makes use of the Checksum Generator on the Sender side and Checksum Checker on the Receiver side.

RESOURCE: Turbo C

PROGRAM LOGIC or APPROACH : The given problem can be divided into two

following parts:

- Generating the Checksum value of the sender’s message can be done using the following

- steps:

      - Divide the message into the binary strings of the given block size.

      - All the binary strings are added together to get the sum.

      - The One’s Complement of the binary string representing the sum is the required

- checksum value.

- Check if the value of the received message (i.e, rec_message + senders_checksum) is equal to 0.

      - The checksum of the received message can be calculated similarly to the checksum

- calculated in the above process.

      - If the checksum value is 0, the message is transmitted properly with no errors

- otherwise, some error has occurred during the transmission.

Below is the implementation of the above approach:

// C++ implementation of the above approach #include <bits/stdc++.h> using namespace std; // Function to find the One's complement // of the given binary string string Ones_complement(string data) { for (int i = 0; i < data.length(); i++) { if (data[i] == '0') data[i] = '1'; else data[i] = '0'; } return data; } // Function to return the checksum value of // the give string when divided in K size blocks string checkSum(string data, int block_size) { // Check data size is divisible by block_size // Otherwise add '0' front of the data int n = data.length(); if (n % block_size != 0) { int pad_size = block_size - (n % block_size); for (int i = 0; i < pad_size; i++) { data = '0' + data; } } // Binary addition of all blocks with carry string result = ""; // First block of data stored in result variable for (int i = 0; i < block_size; i++) { result += data[i]; } // Loop to calculate the block // wise addition of data for (int i = block_size; i < n; i += block_size) { // Stores the data of the next bloack string next_block = ""; for (int j = i; j < i + block_size; j++) { next_block += data[j]; } // Stores the binary addition of two blocks string additions = ""; int sum = 0, carry = 0; // Loop to calculate the binary addition of // the current two blocls of k size for (int k = block_size - 1; k >= 0; k--) { sum += (next_block[k] - '0') + (result[k] - '0'); carry = sum / 2; if (sum == 0) { additions = '0' + additions; sum = carry; } else if (sum == 1) { additions = '1' + additions; sum = carry; } else if (sum == 2) { additions = '0' + additions; sum = carry; } else { additions = '1' + additions; sum = carry; } } // After binary add of two blocks with carry, // if carry is 1 then apply binary addition string final = ""; if (carry == 1) { for (int l = additions.length() - 1; l >= 0; l--) { if (carry == 0) { final = additions[l] + final; } else if (((additions[l] - '0') + carry) % 2 == 0) { final = "0" + final; carry = 1; } else { final = "1" + final; carry = 0; } } result = final; } else { result = additions; } } // Return One's complements of result value // which represents the required checksum value return Ones_complement(result); } // Function to check if the received message // is same as the senders message bool checker(string sent_message, string rec_message, int block_size) { // Checksum Value of the senders message string sender_checksum = checkSum(sent_message, block_size); // Checksum value for the receivers message string receiver_checksum = checkSum( rec_message + sender_checksum, block_size); // If receivers checksum value is 0 if (count(receiver_checksum.begin(), receiver_checksum.end(), '0') == block_size) { return true; } else { return false; } } // Driver Code int main() { string sent_message = "10000101011000111001010011101101"; string recv_message = "10000101011000111001010011101101"; int block_size = 8; if (checker(sent_message, recv_message, block_size)) { cout << "No Error"; } else { cout << "Error"; } return 0; }

Output

No Error

Time Complexity: O(N)
	Auxiliary Space: O(block_size)

EXPERIMENT NO: 4

NAME OF THE EXPERIMENT: Write a program for Hamming Code generaion for Error

detection and correction

OBJECTIVE: Hamming Code generaion for Error detection and correction.

RESOURCE: Turbo C

PROGRAM LOGIC:

- #include<stdio.h>

- #include<conio.h>

- void main() {

- int data[7],rec[7],i,c1,c2,c3,c;

- printf("this works for message of 4bits in size \nenter message bit

- one by one:  ");

- scanf("%d%d%d%d",&data[0],&data[1],&data[2],&data[4]);

- data[6]=data[0]^data[2]^data[4];

- data[5]=data[0]^data[1]^data[4];

- data[3]=data[0]^data[1]^data[2];

- printf("\nthe encoded bits are given below: \n");

- for (i=0;i<7;i++) {

- printf("%d ",data[i]);

- }

- printf("\nenter the received data bits one by one: ");

- for (i=0;i<7;i++) {

- scanf("%d",&rec[i]);

- }

- c1=rec[6]^rec[4]^rec[2]^rec[0];

- c2=rec[5]^rec[4]^rec[1]^rec[0];

- c3=rec[3]^rec[2]^rec[1]^rec[0];

- c=c3*4+c2*2+c1 ;

- if(c==0) {

- printf("\ncongratulations there is no error: ");

- } else {

- printf("\nerron on the postion: %d\nthe correct message is \n",c);

- if(rec[7-c]==0)

- rec[7-c]=1; else

- rec[7-c]=0;

- for (i=0;i<7;i++) {

- printf("%d ",rec[i]);

- }

- }

- getch();

- }

Output

EXPERIMENT NO: 5

NAME OF THE EXPERIMENT: Cyclic Redundancy Check.

OBJECTIVE: Implement on a data set of characters the three CRC polynomials – CRC12,

CRC 16 and CRC CCIP.

RESOURCE: Turbo C

PROGRAM LOGIC:

CRC method can detect a single burst of length n, since only one bit per column will be changed, a burst of length n+1 will pass undetected, if the first bit is inverted, the last bit is inverted and all other bits are correct. If the block is badly garbled by a long burst or by multiple shorter burst, the probability that any of the n columns will have the correct parity that is 0.5. so the probability of a bad block being expected when it should not be 2 power(-n). This scheme sometimes known as Cyclic Redundancy Code

PROCEDURE: Go to debug -> run or press CTRL + F9 to run the program.

SOURCE CODE:

//PROGRAM FOR CYCLIC REDUNDENCY CHECK

#include<stdio.h> #include<conio.h>

int gen[4],genl,frl,rem[4]; void main()

{

int i,j,fr[8],dupfr[11],recfr[11],tlen,flag; clrscr();

frl=8; genl=4;

printf("enter frame:"); for(i=0;i<frl;i++)

{

scanf("%d",&fr[i]); dupfr[i]=fr[i];

}

printf("enter generator:"); for(i=0;i<genl;i++) scanf("%d",&gen[i]); tlen=frl+genl-1; for(i=frl;i<tlen;i++)

{

dupfr[i]=0;

}

remainder(dupfr); for(i=0;i<frl;i++)

{

recfr[i]=fr[i];

}

for(i=frl,j=1;j<genl;i++,j++)

{

recfr[i]=rem[j];

}

remainder(recfr); flag=0; for(i=0;i<4;i++)

{

if(rem[i]!=0) flag++;

}

if(flag==0)

{

printf("frame received correctly");

}

Else

{

printf("the received frame is wrong");

}

getch();

}

remainder(int fr[])

{

int k,k1,i,j; for(k=0;k<frl;k++)

{

if(fr[k]==1)

{

k1=k; for(i=0,j=k;i<genl;i++,j++)

{

rem[i]=fr[j]^gen[i];

}

for(i=0;i<genl;i++)

{

fr[k1]=rem[i]; k1++;

}

}

}

}

OUTPUT:

Viva Questions:

- What is CRC?

- What is the use of the CRC?

- Name the CRC standards?

- Define Checksum?

- Define generator polynomial?

EXPERIMENT NO: 6

NAME OF THE EXPERIMENT: Program to implement Sliding Window Protocol for

Goback N

OBJECTIVE: Go back n: Sender transmits all frames present in the window that occurs after 		       the error bit including error bit also.

RESOURCE: Turbo C

PROGRAM LOGIC: Go-Back-N protocol, also called Go-Back-N Automatic Repeat reQuest, is a data link layer protocol that uses a sliding window method for reliable and sequential 	delivery of data frames. It is a case of sliding window protocol having to send window size of N and receiving window size of 1.

Working Principle

Go – Back – N ARQ provides for sending multiple frames before receiving the 	acknowledgment for the first frame. The frames are sequentially numbered and a finite number 	of frames. The maximum number of frames that can be sent depends upon the size of the 	sending window. If the acknowledgment of a frame is not received within an agreed upon time 	period, all frames starting from that frame are retransmitted.

CODE IN C:

#include<stdio.h>

int main()

{

int windowsize,sent=0,ack,i;

printf("enter window size\n");

scanf("%d",&windowsize);

while(1)

{

for( i = 0; i < windowsize; i++)

{

printf("Frame %d has been transmitted.\n",sent);

sent++;

if (sent = = windowsize)

break;

}

printf("\nPlease enter the last Acknowledgement received.\n");

scanf("%d",&ack);

if(ack = = windowsize)

break;

else

sent = ack;

}

return 0;

}

OUTPUT:-

Enter window size

8

Frame 0 has been transmitted.

Frame 1 has been transmitted.

Frame 2 has been transmitted.

Frame 3 has been transmitted.

Frame 4 has been transmitted.

Frame 5 has been transmitted.

Frame 6 has been transmitted.

Frame 7 has been transmitted.

Please enter the last Acknowledgement received.

2

Frame 2 has been transmitted.

Frame 3 has been transmitted.

Frame 4 has been transmitted.

Frame 5 has been transmitted.

Frame 6 has been transmitted.

Frame 7 has been transmitted.

Please enter the last Acknowledgement received.

8

**	**	**

EXAMPLE-2 :

#include<stdio.h>
	#include<conio.h>
	void main()
	{
	char sender[50],receiver[50];
	int i,winsize;
 	printf("\n ENTER THE WINDOWS SIZE : ");
	scanf("%d",&winsize);
 	printf("\n SENDER WINDOW IS EXPANDED TO STORE MESSAGE OR 			WINDOW \n");
 	printf("\n ENTER THE DATA TO BE SENT: ");
	fflush(stdin);
	gets(sender);
	for(i=0;i<winsize;i++)
	receiver[i]=sender[i];
	receiver[i]=NULL;
	 printf("\n MESSAGE SEND BY THE SENDER:\n");
	 puts(sender);
 	printf("\n WINDOW SIZE OF RECEIVER IS EXPANDED\n");
	 printf("\n ACKNOWLEDGEMENT FROM RECEIVER \n");
	for(i=0;i<winsize;i++);
	printf("\n ACK:%d",i);
 	printf("\n MESSAGE RECEIVED BY RECEIVER IS : ");
 	puts(receiver);
 	printf("\n WINDOW SIZE OF RECEIVER IS SHRINKED \n");
	getch();
	}

------------------------------------------------------------------------------------------------------------------	Output.…

EXPERIMENT NO: 7

NAME OF THE EXPERIMENT: Program to implement Sliding Window Protocol for

Selective Repeat

OBJECTIVE: Selective Repeat: Sender transmits only that frame which is erroneous or is lost.

RESOURCE: Turbo C

PROGRAM LOGIC: The sliding window protocol is a flow control protocol that allows both

link nodes A and B to send and receive data and acknowledgments simultaneously.

- Here, the sender can send multiple frames without having to wait for acknowledgments.

- If no new data frames are ready for transmission in a specified time, a separate acknowledgment frame is generated to avoid time-out.

- Each outbound frame contains a sequence number ranging from 0 to 2(

Sender Window:

Sender Window is a set of sequence numbers maintained by the sender corresponding to the 	frame sequence numbers of frames sent out but not yet acknowledged.

- The sender can transmit a maximum number of frames before receiving any acknowledgment without blocking (Pipelining).

- All the frames in a sending window can be lost or damaged and hence must be saved in memory or buffer till they are acknowledged.

Receiving Window :

A Receiving Window is a set of sequence numbers that is maintained by the receiver. It allows 	receiving and acknowledging of multiple frames.

- The size of the receiving window is fixed at a specified initial size.

- Any frame received with a sequence number outside the receiving window is discarded.

- The sending and receiving window may not have the same size or any upper or lower limits.

Selective Repeat Protocol :

The selective repeat protocol is an implementation of the sliding window protocol. In the 	selective repeat protocol, both the sender and the receiver maintain a window of outstanding 	and acceptable sequence numbers.

- In SRP, the sender's window size starts at 0 and it grows to some predefined maximum.

- The receiver's window is always fixed in size and equal to the predetermined maximum.

- The receiver has the buffer reserved for each sequence number within its fixed window.

- The sender and the receiver maintain a buffer of their window size.

- If there is an error, the receiver checks the lower edge to the last sequence number before the lost frame sequence number.

- The receiver continues to receive and acknowledge incoming frames.

- The sender maintains a timeout clock for the unacknowledged frame number and retransmits that frame after the timeout.

- The acknowledgment will be piggybacked to the sender. But when there is no traffic in the reverse direction, piggyback is impossible, a special timer will time out for the ACK so that the ACK is sent back as an independent packet. If the receiver suspects that the transmission has an error, it immediately sends back a negative acknowledgment (NAK) to the sender.

Note − SRP works better when the link is very unreliable. Because in this case, retransmission tends to happen more frequently, selectively retransmitting frames is more efficient than retransmitting all of them. In selective repeat protocol, the size of the sender and receiver windows must be at most one-half of 2.

Sender Site Algorithm of Selective Repeat Protocol

begin

frame s; //s denotes frame to be sent

frame t; //t is temporary frame

S_window = power(2,m-1); //Assign maximum window size

SeqFirst = 0; // Sequence number of first frame in window

SeqN = 0; // Sequence number of Nth frame window

while (true) //check repeatedly

do

Wait_For_Event(); //wait for availability of packet

if ( Event(Request_For_Transfer)) then

//check if window is full

if (SeqN–SeqFirst >= S_window) then

doNothing();

end if;

Get_Data_From_Network_Layer();

s = Make_Frame();

s.seq = SeqN;

Store_Copy_Frame(s);

Send_Frame(s);

Start_Timer(s);

SeqN = SeqN + 1;

end if;

if ( Event(Frame_Arrival) then

r = Receive_Acknowledgement();

// Resend frame whose sequence number is with ACK

if ( r.type = NAK) then

if ( NAK_No > SeqFirst && NAK_No < SeqN ) then

Retransmit( s.seq(NAK_No));

Start_Timer(s);

end if

//Remove frames from sending window with positive ACK

else if ( r.type = ACK ) then

Remove_Frame(s.seq(SeqFirst));

Stop_Timer(s);

SeqFirst = SeqFirst + 1;

end if

end if

// Resend frame if acknowledgement haven’t been received

if ( Event(Time_Out)) then

Start_Timer(s);

Retransmit_Frame(s);

end if

end

Receiver Site Algorithm of Selective Repeat Protocol

Begin

frame f;

RSeqNo = 0; // Initialise sequence number of expected frame

NAKsent = false;

ACK = false;

For each slot in receive_window

Mark(slot)=false;

while (true) //check repeatedly

do

Wait_For_Event(); //wait for arrival of frame

if ( Event(Frame_Arrival) then

Receive_Frame_From_Physical_Layer();

if ( Corrupted ( f.SeqNo ) AND NAKsent = false) then

SendNAK(f.SeqNo);

NAKsent = true;

end if

if ( f.SeqNo != RSeqNo AND NAKsent = false ) then

SendNAK(f.SeqNo);

NAKsent = true;

if ( f.SeqNo is in receive_window ) then

if ( Mark(RSeqNo) = false ) then

Store_frame(f.SeqNo);

Mark(RSeqNo) = true;

end if

end if

else

while ( Mark(RSeqNo))

Extract_Data(RSeqNo);

Deliver_Data_To_Network_Layer();

RSeqNo = RSeqNo + 1;

Send_ACK(RSeqNo);

end while

end if

end if

end while

end

EXPERIMENT NO: 10

NAME OF THE EXPERIMENT: Shortest Path.

OBJECTIVE: Implement Dijkstra‘ s algorithm to compute the Shortest path thru a

given graph.

RESOURCE: Turbo C

PROGRAM LOGIC: Dijkstra's algorithm is very similar to Prim's algorithm for minimum spanning tree. Like Prim's MST, we generate a SPT (shortest path tree) with given source as root. We maintain two sets, one set contains vertices included in shortest path tree, and other set includes vertices not yet included in shortest path tree.

PROCEDURE: Go to debug -> run or press CTRL + F9 to run the program.

SOURCE CODE:

// .PROGRAM FOR FINDING SHORTEST PATH FOR A GIVEN GRAPH//

#include<stdio.h> #include<conio.h> void main()

{

int path[5][5],i,j,min,a[5][5],p,st=1,ed=5,stp,edp,t[5],index; clrscr();

printf("enter the cost matrix\n"); for(i=1;i<=5;i++) for(j=1;j<=5;j++) scanf("%d",&a[i][j]); printf("enter the paths\n"); scanf("%d",&p);

printf("enter possible paths\n"); for(i=1;i<=p;i++) for(j=1;j<=5;j++) scanf("%d",&path[i][j]);

for(i=1;i<=p;i++)

{ t[i]=0;

stp=st; for(j=1;j<=5;j++)

{

edp=path[i][j+1]; t[i]=t[i]+a[stp][edp]; if(edp==ed)

break; else stp=edp;

}

}min=t[st]; index=st; for(i=1;i<=p;i++)

{

if(min>t[i])

{

min=t[i]; index=i;

}

}

printf("minimum cost %d",min); printf("\n minimum cost path "); for(i=1;i<=5;i++)

{

printf("--> %d",path[index][i]); if(path[index][i]==ed)

break;

}

getch();

}

Output:

Viva questions:

- Define Dijkstra’s algorithm?

- What is the use of Dijkstra’s algorithm?

- What is path?

- What is minimum cost path?

- How to find shortest path using Dijkstra’s algorithm?

EXPERIMENT NO: 11

NAME OF THE EXPERIMENT: Distance Vector routing.

OBJECTIVE: Obtain Routing table at each node using distance vector routing algorithm for a given subnet.

RESOURCE: Turbo C

PROGRAM LOGIC: Distance Vector Routing Algorithms calculate a best route to reach  a destination based solely on distance. E.g. RIP. RIP calculates the  reach ability based on hop count. It’ s different from link state  algorithms  which  consider  some  other factors like bandwidth and other metrics to reach a  destination. Distance vector routing algorithms are not preferable for complex networks and take longer to converge.

PROCEDURE: Go to debug -> run or press CTRL + F9 to run the program.

SOURCE CODE:

#include<stdio.h> #include<conio.h> struct node

{

unsigned dist[20]; unsigned from[20];

}rt[10];

int main()

{

int dmat[20][20]; int n,i,j,k,count=0; clrscr();

printf("\nEnter the number of nodes : "); scanf("%d",&n);printf("Enter the cost matrix :\n"); for(i=0;i<n;i++)

for(j=0;j<n;j++)

{

scanf("%d",&dmat[i][j]); dmat[i][i]=0; rt[i].dist[j]=dmat[i][j]; rt[i].from[j]=j;

}

Do

{

count=0; for(i=0;i<n;i++) for(j=0;j<n;j++) for(k=0;k<n;k++)

if(rt[i].dist[j]>dmat[i][k]+rt[k].dist[j])

{

rt[i].dist[j]=rt[i].dist[k]+rt[k].dist[j]; rt[i].from[j]=k;count++;

}

}while(count!=0); for(i=0;i<n;i++)

{

printf("\nState value for router %d is \n",i+1); for(j=0;j<n;j++)

{

printf("\nnode %d via %d Distance%d",j+1,rt[i].from[j]+1,rt[i].dist[j]);

}

}

printf(“\n”);

}

Output:

Viva Questions:

- What is routing?

- What is best algorithm among all routing algorithms?

- What is static routing?

- Differences between static and dynamic?

- What is optimality principle?

EXPERIMENT No :12

NAME OF THE EXPERIMENT: Broadcast Tree.

OBJECTIVE: Implement broadcast tree for a given subnet of hosts

RESOURCE: Turbo C

PROGRAM LOGIC:

This technique is widely used because it is simple and easy to understand. The idea of this algorithm is to bui ld a graph of the subnet with each node of the graph representing a router and each arc of the graph representing a communication line.

To choose a route between a given pair of routers the algorithm just finds the broadcast between them on the graph.

PROCEDURE: Go to debug -> run or press CTRL + F9 to run the program.

SOURCE CODE:

// Write a ‘c’ program for Broadcast tree from subnet of host #include<stdio.h>

#include<conio.h> int p,q,u,v,n;

int min=99,mincost=0; int t[50][2],i,j;

int parent[50],edge[50][50]; main()

{

clrscr();

printf("\n Enter the number of nodes"); scanf("%d",&n);

for(i=0;i<n;i++)

{

printf("%c\t",65+i); parent[i]=-1;

}

printf("\n"); for(i=0;i<n;i++)

{

printf("%c",65+i); for(j=0;j<n;j++) scanf("%d",&edge[i][j]);

}

for(i=0;i<n;i++)

{

for(j=0;j<n;j++) if(edge[i][j]!=99)

if(min>edge[i][j])

{

min=edge[i][j]; u=i;

v=j;

}

p=find(u); q=find(v); if(p!=q)

{ t[i][0]=u;

t[i][1]=v; mincost=mincost+edge[u][v]; sunion(p,q);

}

Else

{

t[i][0]=-1;t[i][1]=-1;

}

min=99;

}

printf("Minimum cost is %d\n Minimum spanning tree is\n" ,mincost); for(i=0;i<n;i++)

if(t[i][0]!=-1 && t[i][1]!=-1)

{

printf("%c %c %d", 65+t[i][0],65+t[i][1],edge[t[i][0]][t[i][1]]);printf("\n");

}

getch();

}

sunion(int l,int m)

{

parent[l]=m;

}

find(int l)

{

if(parent[l]>0)

i=parent[i]; return i;

}

Output:

Viva questions:

- What is spanning tree?

- What is broad cast tree?

- What are the advantages of broadcast tree?

- What is flooding?

- What is subnet?

EXPERIMENT No :13

13  ( i  )

NAME OF THE EXPERIMENT: Packet Capture Using Wire Shark

OBJECTIVE: It can capture, dissect, and decode various protocols

RESOURCE: Wireshark, Linux sys admin, Turbo C

PROGRAM LOGIC:

Wireshark is an open source network packet analyzer.

It can capture, dissect, and decode various protocols. This helps Linux sysadmin to 	troubleshoot network issues.

Apart from using wirehshark as a standlone application for debugging network packets, you 	can also write your own extension or plugin using wireshark libraries for your custom 		application.

This tutorial explains how to use wireshark libraries to write custom code to debug network 	packets using a C example program.

	The code explains two parts. First, to capture network packets. Second, to decode packets 	using libwireshark.

As a prerequisite, your system should have both libpcap and wireshark libraries installed.

To capture a packet, refer to How to Perform Packet Sniffing Using Libpcap with C Example

Code.

You can also open an existing pcap file using the following api inside your C program:

pd = pcap_open_offline(pcap_path, errbuf);

Wireshark code uses its own dissection engine (epan module library) to dissect the network 	packets.

The following code shows the necessary steps to initialize it properly.

The functions mentioned below are from the wireshark open source code, which will initialize 	the packet dissection engine, required data structures, variables, GUID mapping, memory 	allocation subsystem, registering all the protocol dissector handles, host name lookup, that are 	necessary for dissection process.

An example

The code below makes use of the libpcap functions to achieve a basic packet capture. After 	capturing the packets, inside the callback function, the length of each packet is printed on 	stdout.

#include <pcap.h>

#include <stdio.h>

#include <stdlib.h>

#include <errno.h>

#include <sys/socket.h>

#include <netinet/in.h>

#include <arpa/inet.h>

#include <netinet/if_ether.h>

#include <netinet/tcp.h>

#include <netinet/ip.h>

#include <string.h>

void callback(u_char *useless,const struct pcap_pkthdr* pkthdr,const u_char*

packet)

{

static int count = 1;

printf("\nPacket number [%d], length of this packet is: %d\n", count++, pkthdr->len);

}

int main(int argc,char **argv)

{

char *dev;

char errbuf[PCAP_ERRBUF_SIZE];

pcap_t* descr;

struct bpf_program fp;        /* to hold compiled program */

bpf_u_int32 pMask;            /* subnet mask */

bpf_u_int32 pNet;             /* ip address*/

pcap_if_t *alldevs, *d;

char dev_buff[64] = {0};

int i =0;

// Check if sufficient arguments were supplied

if(argc != 3)

{

printf("\nUsage: %s [protocol][number-of-packets]\n",argv[0]);

return 0;

}

// Prepare a list of all the devices

if (pcap_findalldevs(&alldevs, errbuf) == -1)

{

fprintf(stderr,"Error in pcap_findalldevs: %s\n", errbuf);

exit(1);

}

// Print the list to user

// so that a choice can be

// made

printf("\nHere is a list of available devices on your system:\n\n");

for(d=alldevs; d; d=d->next)

{

printf("%d. %s", ++i, d->name);

if (d->description)

printf(" (%s)\n", d->description);

else

printf(" (Sorry, No description available for this device)\n");

}

// Ask user to provide the interface name

printf("\nEnter the interface name on which you want to run the packet sniffer : ");

fgets(dev_buff, sizeof(dev_buff)-1, stdin);

// Clear off the trailing newline that

// fgets sets

dev_buff[strlen(dev_buff)-1] = '';

// Check if something was provided

// by user

if(strlen(dev_buff))

{

dev = dev_buff;

printf("\n ---You opted for device [%s] to capture [%d] packets---\n\n Starting capture...",dev, (atoi)(argv[2]));

}

// If something was not provided

// return error.

if(dev == NULL)

{

printf("\n[%s]\n", errbuf);

return -1;

}

// fetch the network address and network mask

pcap_lookupnet(dev, &pNet, &pMask, errbuf);

// Now, open device for sniffing

descr = pcap_open_live(dev, BUFSIZ, 0,-1, errbuf);

if(descr == NULL)

{

printf("pcap_open_live() failed due to [%s]\n", errbuf);

return -1;

}

// Compile the filter expression

if(pcap_compile(descr, &fp, argv[1], 0, pNet) == -1)

{

printf("\npcap_compile() failed\n");

return -1;

}

// Set the filter compiled above

if(pcap_setfilter(descr, &fp) == -1)

{

printf("\npcap_setfilter() failed\n");

exit(1);

}

// For every packet received, call the callback function

// For now, maximum limit on number of packets is specified

// by user.

pcap_loop(descr,atoi(argv[2]), callback, NULL);

printf("\nDone with packet sniffing!\n");

return 0;

}

Output:

In the code above :

- The function pcap_findalldevs() is used to fetch a list of all available interface devices. This list can be shown to the user so that the intended interface can be selected to sniff packets on. Please note that these is exists a function pcap_lookupdev() that also returns an interface device but the problem with this function is that it returns the first available non loop-back device. So in case I am using wireless network connection and the interface device for my connection is ‘wlan0’ but pcap_lookupdev() function would still return ‘eth0’ as it encounters this interface first. So using pcap_findalldevs() is a better option as it  produces a list of interface devices to choose from.

- The list returned by the function pcap_findalldevs() is given to user and the user’s input is taken from stdin.

- Then the function pcap_lookupnet() is used to fetch the ip address and network mask.

- Through the function pcap_open_live() the pcap library is initialized with the interface device selected.

- Through pcap_compile() function , we can compile any filter on protocol etc set by the user.

- Through pcap_setfilter(), this filter is applied.

- Finally through function pcap_loop() the library starts packet capture on the selected device with the filter applied and with every relevant packet captured, the callback function is called.

Here is the output of above program :

$ sudo ./pcap tcp 10

[sudo] password for himanshu:

Here is a list of available devices on your system:

1. eth0 (Sorry, No description available for this device)

2. wlan0 (Sorry, No description available for this device)

3. usbmon1 (USB bus number 1)

4. usbmon2 (USB bus number 2)

5. usbmon3 (USB bus number 3)

6. usbmon4 (USB bus number 4)

7. usbmon5 (USB bus number 5)

8. usbmon6 (USB bus number 6)

9. usbmon7 (USB bus number 7)

10. any (Pseudo-device that captures on all interfaces)

11. lo (Sorry, No description available for this device)

Enter the interface name on which you want to run the packet sniffer : wlan0

---You opted for device [wlan0] to capture [10] packets---

Starting capture...

Packet number [1], length of this packet is: 496

Packet number [2], length of this packet is: 66

Packet number [3], length of this packet is: 357

Packet number [4], length of this packet is: 66

Packet number [5], length of this packet is: 238

Packet number [6], length of this packet is: 66

Packet number [7], length of this packet is: 403

Packet number [8], length of this packet is: 66

Packet number [9], length of this packet is: 121

Packet number [10], length of this packet is: 66

Done with packet sniffing!

If you are not executing the above program as root, you should use sudo to run the program as the actions done by libpcap library requires super user privileges.

EXPERIMENT No : 13

13 ( i i )

ii) Start Capturing

The following methods can be used to start capturing packets with Wireshark:

- You can double-click on an interface in the welcome screen.

- You can select an interface in the welcome screen, then select Capture → Start or click the first

- toolbar button.

- You can get more detailed information about available interfaces using Section 4.5, “The

- “Capture  Options” Dialog Box” (Capture → Options…​).

- If you already know the name of the capture interface you can start Wireshark from the

- command  line:

$ wireshark -i eth0 -k

This will start Wireshark capturing on interface eth0. More details can be found at Section 11.2,

“Start Wireshark from the command line”.

EXPERIMENT No : 13

13 ( i ii )

Viewing Packets You Have Captured

Once you have captured some packets or you have opened a previously saved capture file, you 	can view the packets that are displayed in the packet list pane by simply clicking on a packet in 	the packet list pane, which will bring up the selected packet in the tree view and byte view panes.

You can then expand any part of the tree to view detailed information about each protocol in 	each packet. Clicking on an item in the tree will highlight the corresponding bytes in the byte 	view.

An example with a TCP packet selected is shown in Figure 6.1, “Wireshark with a TCP packet 	selected for viewing”. It also has the Acknowledgment number in the TCP header selected,

which shows up in the byte view as the selected bytes.

Figure 6.1. Wireshark with a TCP packet selected for viewing

You can also select and view packets the same way while Wireshark is capturing if you selected 	“Update list of packets in real time” in the “Capture Preferences” dialog box.

In addition you can view individual packets in a separate window as shown in Figure 6.2, 	“Viewing a packet in a separate window”. You can do this by double-clicking on an item in the 	packet list or by selecting the packet in which you are interested in the packet list pane and 	selecting View → Show Packet in New Window. This allows you to easily compare two or more 	packets, even across multiple files.

Figure 6.2. Viewing a packet in a separate window

Along with double-clicking the packet list and using the main menu there are a number of other 	ways to open a new packet window:

EXPERIMENT No : 13

13 ( i v)

(iv) The “Statistics” Menu

The Wireshark Statistics menu contains the fields shown in Table 3.9, “Statistics menu items”.

Figure 3.9. The “Statistics” Menu

Each menu item brings up a new window showing specific statistics.

Table 3.9. Statistics menu items

| Menu Item | Accelerator | Description |
| --- | --- | --- |
| Capture File Properties |  | Show information about the capture file, see Section 8.2, “The “Capture File Properties” Dialog”. |
| Resolved Addresses |  | See Section 8.3, “Resolved Addresses” |
| Protocol Hierarchy |  | Display a hierarchical tree of protocol statistics, see Section 8.4, “The “Protocol Hierarchy” Window”. |
| Conversations |  | Display a list of conversations (traffic between two endpoints), see Section 8.5.1, “The “Conversations” Window”. |
| Endpoints |  | Display a list of endpoints (traffic to/from an address), see Section 8.6.1, “The “Endpoints” Window”. |
| Packet Lengths |  | See Section 8.7, “Packet Lengths” |
| I/O Graphs |  | Display user specified graphs (e.g., the number of packets in the course of time), see Section 8.8, “The “I/O Graphs” Window”. |
| Service Response Time |  | Display the time between a request and the corresponding response, see Section 8.9, “Service Response Time”. |
| DHCP (BOOTP) |  | See Section 8.10, “DHCP (BOOTP) Statistics” |
| NetPerfMeter |  | See Section 8.11, “NetPerfMeter Statistics” |
| ONC-RPC Programs |  | See Section 8.12, “ONC-RPC Programs” |
| 29West |  | See Section 8.13, “29West” |
| ANCP |  | See Section 8.14, “ANCP” |
| BACnet |  | See Section 8.15, “BACnet” |
| Collectd |  | See Section 8.16, “Collectd” |
| DNS |  | See Section 8.17, “DNS” |
| Flow Graph |  | See Section 8.18, “Flow Graph” |
| HART-IP |  | See Section 8.19, “HART-IP” |
| HPFEEDS |  | See Section 8.20, “HPFEEDS” |
| HTTP |  | HTTP request/response statistics, see Section 8.21, “HTTP Statistics” |
| HTTP2 |  | See Section 8.22, “HTTP2” |
| Sametime |  | See Section 8.23, “Sametime” |
| TCP Stream Graphs |  | See Section 8.24, “TCP Stream Graphs” |
| UDP Multicast Streams |  | See Section 8.25, “UDP Multicast Streams” |
| Reliable Server Pooling (RSerPool) |  | See Section 8.26, “Reliable Server Pooling (RSerPool)” |
| F5 |  | See Section 8.27, “F5” |
| IPv4 Statistics |  | See Section 8.28, “IPv4 Statistics” |
| IPv6 Statistics |  | See Section 8.29, “IPv6 Statistics” |

EXPERIMENT No : 14

NAME OF THE EXPERIMENT: How to Run a Simple Nmap Scan.

OBJECTIVE: Ensuring that your router is protected from unwanted intruders is one of the

foundations of a secure network.

RESOURCE: Nmap Installer, Linux, Windows

PROGRAM LOGIC:  Ensuring that your router is protected from unwanted intruders is one of the foundations of a secure network. One of the basic tools for this job is Nmap, or Network Mapper. This program will scan a target and report which ports are open and which are closed, among other things. Security specialists use this program to test the security of a network. To learn how to use it yourself, see Step 1 below.

How to Run a Simple Nmap Scan

- Download the installer.

- Run the installer.

- Open NMap/Zenmap.

- Enter the target address.

- Choose a profile.

- Click Scan.

- Review the results.

Using ZenmapDownload Article

1. Download the Nmap installer. This can be found for free from the developer’s website. It 	is highly recommended that you download directly from the developer to avoid any potential 	viruses or fake files. Downloading the Nmap installer includes Zenmap, the graphical interface 	for Nmap which makes it easy for newcomers to perform scans without having to learn 	command lines.

- The Zenmap program is available for Windows, Linux, and Mac OS X. You can find the installation files for all operating systems on the Nmap website.

2. Install Nmap. Run the installer once it is finished downloading. You will be asked which 	components you would like to install. In order to get the full benefit of Nmap, keep all of these 	checked. Nmap will not install any adware or spyware.

- Run the "Nmap – Zenmap" GUI program. If you left your settings at default during

- installation, you should be able to see an icon for it on your desktop. If not, look in your Start menu. Opening Zenmap will start the program.

4.  Enter in the target for your scan. The Zenmap program makes scanning a fairly simple
 	     process. The first step to running a scan is choosing your target. You can enter a domain

(example.com), an IP address (127.0.0.1), a network (192.168.1.0/24), or a combination of

those.

- Depending on the intensity and target of your scan, running an Nmap scan may be against the terms of your internet service provider, and may land you in hot water. Always check your local laws and your ISP contract before performing Nmap scans on targets other than your own network.

- 5.Choose your Profile. Profiles are preset groupings of modifiers that change what is scanned. The profiles allow you to quickly select different types of scans without having to type in the modifiers on the command line. Choose the profile that best fits your needs:[1]

- Intense scan - A comprehensive scan. Contains Operating System (OS) detection, version detection, script scanning, traceroute, and has aggressive scan timing. This is considered an intrusive scan.

- Ping scan - This scan simply detects if the targets are online, it does not scan any ports.

- Quick scan - This is quicker than a regular scan due to aggressive timing and only scanning select ports.

- Regular scan - This is the standard Nmap scan without any modifiers. It will return ping and return open ports on the target.

- Click Scan to start scanning. The active results of the scan will be displayed in the Nmap Output tab. The time the scan takes will depend on the scan profile you chose, the physical distance to the target, and the target’s network configuration.

- Read your results. Once the scan is finished, you’ll see the message "Nmap done" at the bottom of the Nmap Output tab. You can now check your results, depending on the type of scan you performed. All of the results will be listed in the main Nmap Output tab, but you can use the other tabs to get a better look at specific data.[2]

- Ports/Hosts - This tab will show the results of your port scan, including the services for those ports.

- Topology - This shows the traceroute for the scan you performed. You can see how many hops your data goes through to reach the target.

- Host Details - This shows a summary of your target learned through scans, such as the number of ports, IP addresses, hostnames, operating systems, and more.

  - Scans - This tab stores the commands of your previously-run scans. This allows you to quickly re-scan with a specific set of parameters.

- METHOD-2

- Using the Command Line

1. Install Nmap. Before using Nmap, you will need to install it so that you can run it from the

command line of your operating system. Nmap is small and available for free from the

developer. 	Follow the instructions below for your operating system:

- Linux - Download and install Nmap from your repository. Nmap is available through most of the major Linux repositories. Enter in the command below based on your distribution:

- Red Hat, Fedora, SUSE
rpm -vhU http://nmap.org/dist/nmap-6.40-1.i386.rpm (32-bit) OR
rpm -vhU http://nmap.org/dist/nmap-6.40-1.x86_64.rpm (64-bit)

    - Debian, Ubuntu
sudo apt-get install nmap

- Windows - Download the Nmap installer. This can be found for free from the developer’s website. It is highly recommended that you download directly from the developer to avoid any potential viruses or fake files. Using the installer allows you to quickly install the command line Nmap tools without having to worry about extracting to the right folder.

- If you don’t want the Zenmap graphical user interface, you can uncheck it during the installation process.

- Mac OS X – Download the Nmap disk image. This can be found for free from the developer’s website. It is highly recommended that you download directly from the developer to avoid any potential viruses or fake files. Use the included installer to install Nmap on your system. Nmap requires OS X 10.6 or later.

2. Open your command line. Nmap commands are run from the command line, and the

results are displayed beneath the command. You can use variables to modify the scan.

You can run the scan from any directory on the command line.

- Linux - Open the terminal if you are using a GUI for your Linux distribution. The location of the terminal varies by distribution

- Windows - This can be accessed by pressing the Windows key + R and then typing "cmd" into the Run field. Windows 8 users can press Windows key + X and select Command Prompt from the menu. You can run an Nmap scan from any directory.

- Mac OS X - Open the Terminal application located in the Utility subfolder of your Applications folder.

3.  Run a scan of you target’s ports. To start a basic scan, type nmap <target>. This will

ping the target and scan the ports. This is an easily-detected scan. The results will be

displayed on your screen. You may need to scroll back up to see all of the results.

- Depending on the intensity and target of your scan, running an Nmap scan may be against the terms of your internet service provider, and may land you in hot water. Always check your local laws and your ISP contract before performing Nmap scans on targets other than your own network.

- Run a modified scan. You can use command line variables to change the parameters

- of the scan, resulting in more detailed or less detailed results. Changing the scan

- variables will change the intrusiveness of the scan. You can add multiple variables by

- placing a space between each one. Variables come before the target: nmap <variable>

- <variable>  <target>[3]

- -sS - This is a SYN stealth scan. It is less detectable than a standard scan, but may take longer. Many modern firewalls can detect an –sS scan.

- -sn - This is a ping scan. This will disable port scanning, and will only check to see if the host is online.

- -O - This is an operating system scan. The scan will attempt to determine the operating system of the target.

- -A - This variable enables several of the most commonly used scans: OS detection, version detection, script scanning, and traceroute.

- -F - This enables fast mode, and will reduce the number of ports scanned.

- -v - This will show more information in your results, making them easier to read.

5.  Output the scan to an XML file. You can set your scan results to be outputted as an

XML file so that you can easily read them in any web browser. To do this, you will need to

use the -oX variable, as well as set a filename for the new XML file. A completed command

would look similar to nmap –oX Scan Results.xml <target>.

- The XML file will be saved to whatever your current working location is.

EXPERIMENT No : 15

NAME OF THE EXPERIMENT: Operating System Detection using Nmap

OBJECTIVE: Nmap uses TCP/IP stack fingerprinting for OS detection

RESOURCE: Nmap Installer, Linux, Windows

PROGRAM LOGIC:  OS Detection

One of Nmap's best-known features is remote OS detection using TCP/IP stack fingerprinting. 	Nmap sends a series of TCP and UDP packets to the remote host and examines practically 	every bit in the responses. After performing dozens of tests such as TCP ISN sampling, TCP 	options support and ordering, IP ID sampling, and the initial window size check, Nmap 	compares the results to its nmap-os-db database of more than 2,600 known OS fingerprints and 	prints out the OS details if there is a match. Each fingerprint includes a freeform textual 	description of the OS, and a classification which provides the vendor name (e.g. Sun), 	underlying OS (e.g. Solaris), OS generation (e.g. 10), and device type (general purpose, router, 	switch, game console, etc). Most fingerprints also have a Common Platform Enumeration 	(CPE) representation, like cpe:/o:linux:linux_kernel:2.6.

If Nmap is unable to guess the OS of a machine, and conditions are good (e.g. at least one open 	port and one closed port were found), Nmap will provide a URL you can use to submit the 	fingerprint if you know (for sure) the OS running on the machine. By doing this you contribute 	to the pool of operating systems known to Nmap and thus it will be more accurate for everyone.

OS detection enables some other tests which make use of information that is gathered during 	the process anyway. One of these is TCP Sequence Predictability Classification. This measures 	approximately how hard it is to establish a forged TCP connection against the remote host. It is 	useful for exploiting source-IP based trust relationships (rlogin, firewall filters, etc) or for 	hiding the source of an attack. This sort of spoofing is rarely performed any more, but many 	machines are still vulnerable to it. The actual difficulty number is based on statistical sampling 	and may fluctuate. It is generally better to use the English classification such as “worthy 	challenge” or “trivial joke”. This is only reported in normal output in verbose (-v) mode. When 	verbose mode is enabled along with -O, IP ID sequence generation is also reported. Most 	machines are in the “incremental” class, which means that they increment the ID field in the IP 	header for each packet they send. This makes them vulnerable to several advanced information 	gathering and spoofing attacks.

Another bit of extra information enabled by OS detection is a guess at a target's uptime. This 	uses the TCP timestamp option (RFC 1323) to guess when a machine was last rebooted. The 	guess can be inaccurate due to the timestamp counter not being initialized to zero or the 	counter overflowing and wrapping around, so it is printed only in verbose mode.

OS detection is covered in Chapter 8, Remote OS Detection.

OS detection is enabled and controlled with the following options:

-O (Enable OS detection)

Enables OS detection, as discussed above. Alternatively, you can use -A to enable OS 	detection along with other things.

--osscan-limit (Limit OS detection to promising targets)

OS detection is far more effective if at least one open and one closed TCP port are found. Set 	this option and Nmap will not even try OS detection against hosts that do not meet this criteria. 	This can save substantial time, particularly on -Pn scans against many hosts. It only matters 	when OS detection is requested with -O or -A.

--osscan-guess; --fuzzy (Guess OS detection results)

When Nmap is unable to detect a perfect OS match, it sometimes offers up near-matches as 	possibilities. The match has to be very close for Nmap to do this by default. Either of these 	(equivalent) options make Nmap guess more aggressively. Nmap will still tell you when an 	imperfect match is printed and display its confidence level (percentage) for each guess.

--max-os-tries (Set the maximum number of OS detection tries against a target)

When Nmap performs OS detection against a target and fails to find a perfect match, it usually 	repeats	the attempt. By default, Nmap tries five times if conditions are favorable for OS 	fingerprint submission, and twice when conditions aren't so good.

Specifying a lower --max-os-tries value (such as 1) speeds Nmap up, though you miss out on 	retries which could potentially identify the OS. Alternatively, a high value may be set to allow 	even more retries when conditions are favorable. This is rarely done, except to generate better 	fingerprints for submission and integration into the Nmap OS database.

Introduction

While Nmap has supported OS detection since 1998, this chapter describes the 2nd generation 	system released in 2006.

Reasons for OS Detection

While some benefits of discovering the underlying OS and device types on a network are 	obvious, others are more obscure. This section lists the top reasons I hear for discovering this 	extra information.

Determining vulnerability of target hosts

It is sometimes very difficult to determine remotely whether an available service is susceptible 	or patched for a certain vulnerability. Even obtaining the application version number doesn't 	always help, since OS distributors often back-port security fixes without changing the version 	number. The surest way to verify that a vulnerability is real is to exploit it, but that risks 	crashing the service and can lead to wasted hours or even days of frustrating exploitation 	efforts if the service turns out to be patched.

OS detection can help reduce these false positives. For example, the Rwho daemon on unpatched Sun Solaris 7 through 9 may be remotely exploitable (Sun alert #57659). Remotely 	determining vulnerability is difficult, but you can rule it out by finding that a target system is 	running Solaris 10.

Taking this from the perspective of a systems administrator rather than a pen-tester, imagine 	you run a large Sun shop when alert #57659 comes out. Scan your whole network with OS 	detection to find machines which need patching before the bad guys do.

Tailoring exploits

Even after you discover a vulnerability in a target system, OS detection can be helpful in 	exploiting it. Buffer overflows, format-string exploits, and many other vulnerabilities often 	require custom-tailored shellcode with offsets and assembly payloads generated to match the 	target OS and hardware architecture. In some cases, you only get one try because the service 	crashes if you get the shellcode wrong. Use OS detection first or you may end up sending 	Linux shellcode to a FreeBSD server.

Network inventory and support

While it isn't as exciting as busting root through a specially crafted format string exploit, there 	are many administrative reasons to keep track of what is running on your network. Before you 	renew 	that IRIX support contract for another year, scan to see if anyone still uses such 	machines. An inventory can also be useful for IT budgeting and ensuring that all company 	equipment is accounted for.

Detecting unauthorized and dangerous devices

With the ubiquity of mobile devices and cheap commodity networking equipment, companies 	are increasingly finding that employees are extending their networks in undesirable ways. 	They may install a $20 wireless access point (WAP) in their cubicle without realizing (or 	caring) that they just opened up the protected corporate network to potential attackers in the 	parking lot or nearby buildings. WAPs can be so dangerous that Nmap has a special category 	for detecting them, as demonstrated in the section called “SOLUTION: Detect Rogue Wireless 	Access Points on an Enterprise Network”. Users may also cause sysadmins grief by connecting 	insecure and/or worm-infected laptops to the corporate network. Regular scanning can detect 	unauthorized devices for investigation and containment.

Social engineering

Another possible use is social engineering. Lets say that you are scanning a target company 	and Nmap reports a “Datavoice TxPORT PRISM 3000 T1 CSU/DSU 6.22/2.06”. You could 	call up the target pretending to be Datavoice support and discuss some issues with their 	PRISM 3000. Tell them you are about to announce a big security hole, but are first providing 	the patch to valued customers. Some naive administrators might assume that only an 	authorized engineer from Datavoice would know so much about their CSU/DSU. Of course the 	patch you send them is a Trojan horse that gives you remote access to sniff and traipse through 	their network. Be sure to read the rest of this chapter for detection accuracy and verification 	advice before trying this. If you guess the target system wrong and they call the police, that 	will be an embarrassing story to tell your cellmates.

Usage and Examples

The inner workings of OS detection are quite complex, but it is one of the easiest features to 	use. Simply add -O to your scan options. You may want to also increase the verbosity with -	v for even more OS-related details. This is shown in Example 8.1.

Example 8.1. OS detection with verbosity (-O -v)

# nmap -O -v scanme.nmap.org

Starting Nmap ( https://nmap.org )

Nmap scan report for scanme.nmap.org (74.207.244.221)

Not shown: 994 closed ports

PORT      STATE    SERVICE

22/tcp    open     ssh

80/tcp    open     http

646/tcp   filtered ldp

1720/tcp  filtered H.323/Q.931

9929/tcp  open     nping-echo

31337/tcp open     Elite

Device type: general purpose

Running: Linux 2.6.X

OS CPE: cpe:/o:linux:linux_kernel:2.6.39

OS details: Linux 2.6.39

Uptime guess: 1.674 days (since Fri Sep  9 12:03:04 2011)

Network Distance: 10 hops

TCP Sequence Prediction: Difficulty=205 (Good luck!)

IP ID Sequence Generation: All zeros

Read data files from: /usr/local/bin/../share/nmap

Nmap done: 1 IP address (1 host up) scanned in 5.58 seconds

Raw packets sent: 1063 (47.432KB) | Rcvd: 1031 (41.664KB)

Including the -O -v options caused Nmap to generate the following extra line items:

Device type

All fingerprints are classified with one or more high-level device types, such 		as router, printer, firewall, or (as in this case) general purpose. These are further described 	in the section called “Device and OS classification (Class lines)”. Several device types may be 	shown, in which case they will be separated with the pipe symbol as in “Device Type: 	router|firewall”.

Running

This field is also related to the OS classification scheme described in the section called 	“Device and OS classification (Class lines)”. It shows the OS Family (Linux in this case) and 	OS generation (2.6.X) if available. If there are multiple OS families, they are separated by 	commas. When Nmap can't narrow down OS generations to one specific choice, options are 	separated by the pipe symbol ('|') Examples include OpenBSD 3.X, NetBSD 	3.X|4.X and Linux 2.4.X|2.5.X|2.6.X.

If Nmap finds too many OS families to print concisely, it will omit this line. When there are no 	perfect matches, Nmap changes the field to Running (JUST GUESSING) and adds an accuracy 	percentage (100% is a perfect match) in parentheses after each candidate family name. If no 	fingerprints are close matches, the line is omitted.

OS CPE

This shows a Common Platform Enumeration (CPE) representation of the operating system 	when available. It may also have a CPE representation of the hardware type. OS CPE begins 	with cpe:/o and hardware CPE begins with cpe:/h. For more about CPE see the section called 	“Common Platform Enumeration (CPE)”.

OS details

This line gives the detailed description for each fingerprint that matches. While the Device 	type and Running lines are from predefined enumerated lists that are easy to parse by a 	computer, the OS details line contains free-form data which is useful to a human reading the 	report. This can include more exact version numbers, device models, and architectures specific 	to a given fingerprint. In this example, the only matching fingerprint was Linux 2.6.20-1 	(Fedora Core 5). When there are multiple exact matches, they are comma-separated. If there 	aren't any perfect matches, but some close guesses, the field is renamed Aggressive OS 	guesses and fingerprints are shown followed by a percentage in parentheses which specifies 	how close each match was.

Uptime guess

As part of OS detection, Nmap receives several SYN/ACK TCP packets in a row and checks 	the headers for a timestamp option. Many operating systems use a simple counter for this 	which starts at zero at boot time then increments at a constant rate such as twice per second. 	By looking at several responses, Nmap can determine the current values and rate of increase. 	Simple linear extrapolation determines boot time. The timestamp algorithm is used for OS 	detection too (see the section called “TCP timestamp option algorithm (TS)”) since the 	increment rate on different systems varies from 2 Hz to 1,000 Hz.

The uptime guess is labeled a “guess” because various factors can make it completely 	inaccurate. Some operating systems do not start the timestamp counter at zero, but initialize it 	with a random value, making extrapolation to zero meaningless. Even on systems using a 	simple counter starting at zero, the counter eventually overflows and wraps around. With a 	1,000 Hz counter increment rate, the counter resets to zero roughly every 50 days. So a host 	that has been up for 102 days will appear to have been up only two days. Even with these 	caveats, the uptime guess is accurate much of the time for most operating systems, so it is 	printed when available, but only in verbose mode. The uptime guess is omitted if the target 	gives zeros or no timestamp options in its SYN/ACK packets, or if it does not reply at all. The 	line is also omitted if Nmap cannot discern the timestamp increment rate or it seems suspicious 	(like a 	30-year uptime).

Network Distance

A side effect of one of the OS detection tests allows Nmap to compute how many routers are 	between it and a target host. The distance is zero when you are scanning localhost, and one for 	a machine on the same network segment. Each additional router on the path adds one to the 	hop count. The Network Distance line is not printed in this example, since Nmap omits the line 	when it cannot be computed (no reply to the relevant probe).

TCP Sequence Prediction

Systems with poor TCP initial sequence number generation are vulnerable to blind TCP 	spoofing attacks. In other words, you can make a full connection to those systems and send 	(but not receive) data while spoofing a different IP address. The target's logs will show the 	spoofed IP, and you can take advantage of any trust relationship between them. This attack 	was all the rage in the mid-nineties when people commonly used rlogin to allow logins to their 	account without any password from trusted IP addresses. Kevin Mitnick is alleged to have 	used this attack to break into Tsutomu Shimomura's computers in December 1994.

The good news is that hardly anyone uses rlogin anymore, and many operating systems have 	been fixed to use unpredictable initial sequence numbers as proposed by RFC 1948. For these 	reasons, this line is only printed in verbose mode. Sadly, many vendors still ship vulnerable 	operating systems and devices. Even the fixed ones often vary in implementation, which leaves 	them valuable for OS detection purposes. The class describes the ISN generation algorithm 	used by the target, and difficulty is a rough estimate of how hard the system makes blind IP 	spoofing (0 is the easiest). The parenthesized comment is based on the difficulty index and 	ranges from Trivial joke to Easy, Medium, Formidable, Worthy challenge, and finally Good l	uck! Further details about sequence tests are provided in the section called “TCP ISN greatest 	common divisor (GCD)”.

While the rlogin family is mostly a relic of the past, clever attackers can still find effective uses 	for blind TCP spoofing. For example, it allows for spoofed HTTP requests. You don't see the 	results, but just the URL (POST or GET request) can have dramatic side effects. The spoofing 	allows attackers to hide their identity, frame someone else, or exploit IP address restrictions.

IP ID sequence generation

Many systems unwittingly give away sensitive information about their traffic levels based on 	how they generate the lowly 16-bit ID field in IP packets. This can be abused to spoof a port 	scan against other systems and for other mischievous purposes discussed in the section called 	“TCP Idle Scan (-sI)”. This field describes the ID generation algorithm that Nmap was able to 	discern. More information on how it classifies them is available in the section called “IP ID 	sequence generation algorithm (TI, CI, II)”. Note that many systems use a different IP ID 	space for each host they communicate with. In that case, they may appear vulnerable (such as 	showing the Incremental class) while still being secure against attacks such as the idle scan. 	For this reason, and because the issue is rarely critical, the IP ID sequence generation line is 	only printed in verbose mode. If Nmap does not receive sufficient responses during OS 	detection, it will omit the whole line. The best way to test whether a host is vulnerable to being 	an idle scan zombie is to test it with -sI.

While TCP fingerprinting is a powerful method for OS detection, interrogating open ports for 	clues is another effective approach. Some applications, such as Microsoft IIS, only run on a 	single platform (thus giving it away), while many other apps divulge their platform in overly 	verbose banner messages. Adding the -sV option enables Nmap version detection, which is 	trained to look for these clues (among others). In Example 8.2, Nmap catches the platform 	details from an FTP server.

Example 8.2. Using version scan to detect the OS

# nmap -sV -O -v 129.128.X.XX

Starting Nmap ( https://nmap.org )

Nmap scan report for [hostname] (129.128.X.XX)

Not shown: 994 closed ports

PORT      STATE    SERVICE      VERSION

21/tcp    open     ftp          HP-UX 10.x ftpd 4.1

22/tcp    open     ssh          OpenSSH 3.7.1p1 (protocol 1.99)

111/tcp   open     rpc

445/tcp   filtered microsoft-ds

1526/tcp  open     oracle-tns   Oracle TNS Listener

32775/tcp open     rpc

No exact OS matches for host

TCP Sequence Prediction: Class=truly random

Difficulty=9999999 (Good luck!)

IP ID Sequence Generation: Incremental

Service Info: OS: HP-UX

In this example, the line “No exact OS matches for host” means that TCP/IP fingerprinting 	failed to find an exact match. Fortunately, the Service Info field a few lines down discloses that 	the OS is HP-UX. If several operating systems were detected (which can happen with NAT 	gateway boxes that redirect ports to several different machines), the field would be OSs and 	the values would be comma separated. The Service Info line can also contain hostnames and 	device types found during the version scan. The focus of this chapter is on TCP/IP 	fingerprinting though, since version detection was covered in Chapter 7, Service and 	Application Version Detection.

With two effective OS detection methods available, which one should you use? The best 	answer is usually both. In some cases, such as a proxy firewall forwarding to an application on 	another host, the answers may legitimately differ. TCP/IP fingerprinting will identify the proxy 	while version scanning will generally detect the server running the proxied application. Even 	when no proxying or port forwarding is involved, using both techniques is beneficial. If they 	come out the same, that makes the results more credible. If they come out wildly different, 	investigate further to determine what is going on before relying on either. Since OS and 	version detection go together so well, the -A option enables them both.

OS detection is far more effective if at least one open and one closed TCP port are found. Set 	the --osscan-limit option and Nmap will not even try OS detection against hosts which do not 	meet this criteria. This can save substantial time, particularly on -Pn scans against many hosts. 	You still need to enable OS detection with -O (or -A) for the --osscan-limit option to have any 	effect.

Another OS detection option is --osscan-guess. When Nmap is unable to detect a perfect OS 	match, it sometimes offers up near-matches as possibilities. The match has to be very close for 	Nmap to do this by default. If you specify this option (or the equivalent --fuzzy option), Nmap 	will guess more aggressively. Nmap still tells you when an imperfect match is found and 	display its confidence level (percentage) for each guess.

When Nmap performs OS detection against a target and fails to find a perfect match, it usually 	repeats the attempt. By default, Nmap tries five times if conditions are favorable for OS f	ingerprint submission, and twice when conditions aren't so good. The --max-os-tries option lets 	you change this maximum number of OS detection tries. Lowering it (usually to 1) speeds 	Nmap up, though you miss out on retries which could potentially identify the OS. Alternatively, 	a high value may be set to allow even more retries when conditions are favorable. This is 	rarely done, except to generate better fingerprints for submission and integration into the Nmap 	OS database.

Like just about every other part of Nmap, results ultimately come from the target machine 	itself. While rare, systems are occasionally configured to confuse or mislead Nmap. Several 	programs have even been developed specifically to trick Nmap OS detection (see the section 	called “OS Spoofing”). Your best bet is to use numerous reconnaissance methods to explore a 	network, and don't trust any one of them.

TCP/IP fingerprinting requires collecting detailed information about the target's IP stack. The 	most commonly useful results, such as TTL information, are printed to Nmap output whenever 	they are obtained. Slightly less pertinent information, such as IP ID sequence generation and 	TCP sequence prediction difficulty, is only printed in verbose mode. But if you want all of the 	IP stack details that Nmap collected, you can find it in a compact form called a subject 	fingerprint. Nmap sometimes prints this (for user submission purposes) when it doesn't 	recognize a host. You can also force Nmap to print it (in normal, interactive, and XML formats) 	by enabling debugging with (-d). Then read the section called “Understanding an Nmap 	Fingerprint” to interpret it.